TUTORIAL-Tolerances and Fits

Problem 1: A gear made from steel is to be assembled on a steel shaft with an interference fit. Nominal diameter of the shaft is 60mm and the pitch diameter of the gear is 120mm. The shaft and the gear hole are machined with IT5 and IT6 qualities respectively. If the fit is based on basic shaft system (BSS), determine the nearest standard fit if the minimum interference is 0.013 mm.

Problem 2: A shaft of 30 mm is machined to IT5 quality. A gear produced from cast iron is to be press-fitted over the shaft as shown in the figure below. The outside diameter of the hub of the gear is 50 mm. The minimum torque to be transmitted by the gear shaft assembly is 25 Nm. The maximum tangential stress in the gear hub is required to be 82 Mpa. Modulus of elasticities and poisson’s ratios for cast iron and for steel are Eh=100GPa and Es=207GPa; ϑh=0.211 ϑs=0,3 respectively. Determine the nearest standard fit using Basic Shaft System (BSS) if coefficient of friction, f=0.1 and Length of the fit is 20mm.

ANSWERS

Answer 1

Fitting is based on BSS, therefore fundamental deviation for shaft will be zero,

Shaft=> IT5 => h5 => 600-13-13  (from table A4-2, IT quality is taken as 13 for IT grade 5 and for 60mm diameter.)

Iso standard of the hole will be determined, IT quality is given as 6,

Hole=> IT6 => ?

Imin=0,013 mm = 13µm

Hole=>  60-26-45-19  => from table A4-3, nearest standard is selected as P6.

Result= h5/P6

Answer 2

Nominal dia. =30mm

BSS => h5

IT quality =5

Pressed fit (interference fit)

Hub dia=50 mm

Tmin=25Nm

σt max=82 MPa

f=0,1 (coefficient of friction)

L=20 mm (fit length)

A=2.π.r.L		A=2.π.15.20=1885 mm2

Tmin=pmin.A.f.d2		25000= pmin.1885.0,1.(15)		pmin=8.84 MPa

Lame Equations:

σtmax=pmax.c2+b2c2-b2		82=pmax.252+152252-152		pmax=38,6 MPa

Radial Interference:

δ=δhole+δshaft

δ=bpEhc2+b2c2-b2+μh+bpEsb2+a2b2-a2-μs	shaft is solid, a=0 , materials are different;

Then the equation is reduced to;

δ=bpEhc2+b2c2-b2+μh+bpEs1-μs

Hole (gear)

Eh=100 GPa (cast iron)

µh=0,211 (poisson’s ratio)

b=15mm , c=25mm

Shaft

Es=207 GPa (steel)

µs=0,3 (poisson’s ratio)

a=0 , b=15mm

δmin=15.(8,84)100000252+152252-152+0,211+15.(8,84)2070001-0,3		δmin=0,0035mm

δmax=δminpmaxpmin=0,003538,68,84=0,015mm

Diametral interference:

Imin=2.δmin=2.0,0035=0,007mm=7μm

Imax=2.δmax=2.0,015=0,030mm=30μm

Interference fit for BSS

Shaft;

BSS => h5 => 300-9-9

dU=0 µm

dL=-9 µm

Imin=7 µm

Imax=30 µm

From the figure;

DU=-9-7=-16 µm

DL=Imax=-30 µm

So, the tolerance limits of the hole is 30-16-30

Nearest standard between this values can be selected;

Selection procedures:

- DU≤-16  Imin=7     and   DL≥-30  (Imax=30)

- Since DU and DL are negative, we are looking the grades under K8 and for upper deviation. P6, which is -18 near to -16. It is ok.

- When the IT grade is then added; (-18-13)=-31. -31<-30 but it is ok.

Result = h5/P6 = 0-9/-18-31

Upper and lower limits;

Hole = 29,969mm – 29,982mm

Shaft = 29,910mm – 30,000mm